Line Outage Detection (LOD) Experimental Platform

The LOD experimental platform consists of four components:

MATLAB

Component 1: LOD labeled data generator

MATLAB function lod_labeled_datagen.m

**IMPORTANT**: The function lod_labeled_datagen.m calls on external MATLAB function called LOIF_matrix.m in order to generate LOIF and LODF matrices. Make sure that both MATLAB files are in the same directory.

| Variables | Description |
| --- | --- |
| label | Used to identify file name of Training Data |
| system | Name of the case file in MATPOWER that we are interested in. Ex. 'case118' -> 118 bus system |
| samples | Number of Samples in training data. This is the number of times we simulate each scenario (normal conditions and line outages) |
| output_dir | User Can Specify Where to Save CSV Files. If not provided CSV files will be saved in current directory. |
| hourly_var | Hourly Load Variation that will be used in later experiments when we consider a daily load profile. Ex: 0.5 -> For a certain hour the load at each bus will be half of its original load. Ex: 1.0 -> For a certain hour the load at each bus will be its original load (No hourly Variation) |
| rand_var | Random Load Variation. This is a percentage that the load at every bus can randomly vary from. Ex: 5 -> Random number between -5% to +5% Ex: 1 -> Random number between -1% to +1% |
| rng_seed | The seed of Random Number Generator (used for random load variation). It is useful when trying to replicate the same results from previous training data. The value of rng_seed can be given by user (random number) or if the user enters 0 then the seed for the random number generator is calculated using POSIX time. |
| sol_type | AC/DC Solutions Ex: ‘AC’ -> AC power flow solutions (OPF and PF) Ex: ‘DC’ -> DC Power flow solutions (OPF and PF) |
| otl | Set of Observed Transmission Lines (OTLs) where power flow measurements will be collected from. Ex: [] -> All Lines Ex: 1 -> Line 1 Ex: 1:10 -> Lines 1 – 10 Ex: [1,5,10,15] -> Lines 1, 5, 10, and 15 |

Overview:

In MATLAB we are generating labeled training data on the following scenarios

- System Under Normal Conditions

- All Possible Outage Scenarios (Single-Line Outages)

To do this we use Power Simulation Tool MATPOWER to run optimal power flow (OPF) and Power Flow (PF) solutions.

The OPF solution will find an optimal set of generators to supply load while also keeping the cost of generation low and considering the system constraints. If there is a disconnected line, it will still find a way to provide power to all loads. The PF solution will run the system as is (without fixing generation to supply load), which is what we want, so that we are able to see the effect of each outage as soon as it happens. We do this by getting the generation of the OPF solution and fixing it in our PF solution.

When collecting data for Normal Conditions, we use OPF in order to find optimal dispatch of generators that supply the loads of the system. When collecting data for outage scenarios we use PF solution and fix generation based on the results of OPF solution.

When collecting samples of each scenarios we ensure that the first sample is of the system under base conditions (no load variation). Every sample after that will have a random load variation between –(rand_var)% to +(rand_var)%.

In addition, we check for convergence in OPF solution (For Normal conditions) and PF solutions (For Outages). If the OPF solution for a specific sample does not converge for normal conditions, we do not attempt to collect data on outages since we can’t fix generation in PF solutions. If the PF solution of an outage does not converge in base load sample, we do not attempt to run PF solution for the outage in future samples with load variation.

In MATLAB we are generating the following files:

- RNG_Seeds.csv:

  - This file is updated every time we run lod_labeled_datagen() and saves user parameters that were entered.

- Convergence_Data_{system}_{label}_{sol_type}.csv

  - Convergence Data, this tells which lines converged when running OPF and PF solutions when we attempt to disconnect them.

- Branchdata_{system}_{label}_{sol_type}.csv

  - This is the labeled training data that will be used in python to run our LOD experiments.

- LODF and LOIF Matrices

  - LOIF_{system}.csv: Line Outage Impact Factors

  - LODF_{system}.csv: Line Outage Distribution Factors

PYTHON

With python we execute the following components in LOD Experiment Platform

- Component 2) LOD detectable subset generator (lod_detectable_subsetgen()):

  - The function calculates the subsets for each OTL whose elements are outages that are 100% detectable when looking at the OTL’s power flow measurements using KNN classification.

- Component 3) LOD OTL selection (lod_otl_select()): This function executes the following set of LOD experiments.

  - Greedy Maximum Coverage Problem (MCP): through an iterative process we select a set of OTLs, where in each iteration, we select an OTL with the largest set of uncovered outages based off the detectable subset generator in component 1.

  - High Eta OTL: Based off the results from MCP, we select the same number of OTLs but using high eta lines. Then testing them on the same set of covered outages created from MCP.

  - Random OTL: Based off the results from MCP, we select the same number of OTLs but randomly. Then testing them on the same set of covered outages created from MCP. We do this ten times and get the average KNN performance of the ten tests.

- Component 4) LOD classification experiment execution (lod_exp_exec()):

  - This is the ML model setup and execution for LOD experiments and for detectable subset generator (components 2 and 3).

In python all components are executed under the function called main.

main(k, desired_precision, desired_f1):

From Journal: sa = {x ∈ Ta |  |x| ≥ γ, mindist(x) ≥ β}

| Variables | Description |
| --- | --- |
| k | The number of k-neighbors for KNN classification. |
| desired_precision (for normal conditions) | Used in component 2 to find detectable subsets for each OTL. We want a subset that satisfied the desired precision for normal conditions. Ex. 1.0 We want subsets whose outages will not be confused with normal conditions. The powerflow of OTL during these outages is significantly different than normal powerflow. This will find a minimum gamma value that satisfies desired_precision for each OTL. |
| desired_f1 (Overall KNN Performance) | Used in component 2 to find detectable subsets for each OTL. We want a final subset that satisfies the desired f1-score for overall performance. Ex. 1.0 We want subsets whose outages are also separated by a distance of beta from each other. This will find a minimum beta value that satisfies desired_f1 score for each OTL. |

Component 2: LOD detectable subset generator: Python function lod_detectable_subsetgen()

lod_detectable_subsetgen(k,desired_precision,desired_f1score,f,Ta_sets,Trainin_all):

| Variables | Description |
| --- | --- |
| k | Number of k-neighbors for KNN Classification. (component 4) Determined by user in main function |
| desired_precision | Used in Gamma Selection for detectable subsets, selects outages that won’t be confused with normal conditions based on LOIFs in KNN classifier (component 4). Determined by user in main function |
| desired_f1score | Used in Beta Selection for detectable subsets, select outages whose LOIFs are separated by beta which offers perfect classification in KNN classifier (component 4) Removes Ambiguous Outages Determined by user in main function |
| f | Output File: Used to Save Classification Results Created in main function |
| Ta_sets | LOIFs of all outages for each OTL |
| Training_all | Labeled Training Data, used in KNN classifier (component 4) |

Overview:

From our previous work, we evaluated each OTL by finding a set of detectable outages with the use of LOIFs. LOIF measures the change in power flow of an OTL due to the outage of another in a system. In order to find detectable outages using LOIF, we previously implemented a filtering method where each outage in our detectable set (Sa) must have an LOIF that satisfies the following thresholds

- LOIF >= gamma: We want to get rid of outages that will have little impact to the OTL (power flow does not change after outage/ power redistribution).

After the first stage of filtering, we then have a set of outages that will have a noticeable change in power flow at the OTL. From this smaller set (Sa) of outages, we then look for outages that will have different impacts on the OTL. The change in power flow at OTL should be different for each outage. To do this we find the differences in LOIFs for each outage and select outages whose LOIF's are separated by a minimum distance of beta.

2) mindist(LOIF) >= beta : We find well-separated LOIFs, by sorting all       LOIFs    in descending order and finding the differences in consecutive values abs(Sa[i]- Sa[i-1]).

With these differences we select LOIFs whose difference is greater than or equal to beta. With these two thresholds we find detectable outage set (Sa). This set of outages should be easily classified with our ML model when only looking at the power flow of the OTL.

In our previous work we wanted to see the effect of different gamma and beta values, and we varied each variable by [0.1, 0.4, 0.8]. With Sa we created a observability metric, eta, which we used to select a set of OTLs of sizes [2, 4, 8]. Because eta is directly influenced by gamma and beta, the OTLs that are selected might change which can affect ML performance. In order to find the best set of detectable outages, we would like to find the optimal values of gamma and beta for each OTL that will result in good ML performance.

There are two classification measures that we will use to find the best gamma and beta:

- Precision: True Positives / (True Positves + False Positives)

True Positives: The number samples of a particular class (outage) that the ML model predicted correctly

False Positives: The number of samples that were incorrectly predicted as the class (outage) by the ML model.

**IF Precision is 1.0, then we know that there are no False Positives meaning that all the samples of the particular class were predicted correctly.

-Avg. F1-Score: The harmonic mean of precision and recall, in other words, overall performance metric.

**IF Avg. F1-Score is 1.0, then we know that we got 100% accuracy on classification of all labels (outages).

In order to find the minimum/optimal gamma, we use precision specifically for the class representing normal conditions (0).

- If precision for normal conditions is 1.0, then we know that the outages in the detectable outage set (Sa) provide a significant change in power flow on the OTL

(enough not to be confused with the power flow during normal conditions)

- This is done through an iterative process, where we first start with a very low gamma and increase by small increments with each iteration.

In each iteration we find Sa using the current gamma and run KNN classifier as our ML model to collect a classification report.

From this report, we observe the precision for normal conditions and check if it meets desired value of 1.0 (desired precision)

Once we reach desired precision, we found our optimal/minimum gamma for OTL.

In order to find the minimum/optimal beta, we use Avg. F1-Score (overall performance) for all classes.

-If Avg. F1-Score is 1.0, then we know that the outages in Sa have LOIFs that are separated from each other by a minimum distance of beta and all outages are predicted correctly.

-This is done through an iterative process, where we first start with a very low   beta and increase by small increments with each iteration.

In each iteration, we find Sa using the current beta and run KNN classifier as our ML model to collect a classification report. From this report, we observe the Avg. F1-score and check if it meets desired value of 1.0 (desire f1-score). Once we reach desired f1-score, we found our optimal/minimum beta for OTL.

Component 3: LOD observed transmission line (OTL) selection: Python function lod_otl_select()

lod_otl_select(mgb_df, f, Training_all, LOIF)

| Variables | Descriptions |
| --- | --- |
| mgb_df | DataFrame/CSV containing detectable subsets for all OTL Used For MCP and High Eta Tests |
| f | Output File: Used to Save Classification Results, MCP test, High Eta test, and Random test Results Created in main function |
| Training_all | Labeled Training Data, used in KNN classifier (component 4) |
| LOIF | Line Outage Impact Factors |

MCP Greedy Algorithm

max_coverage_greedy(mgb_df, OTL_num, f, Training_all)

| Variables | Description |
| --- | --- |
| mgb_df | DataFrame/CSV containing detectable subsets for all OTLs |
| OTL_num | Number of OTLs to be selected in OPL, set to high number to consider all outages. |
| f | Output File: Used to Save Classification Results of MCP test Created in main function |
| Training_all | Labeled Training Data, used in KNN classifier (component 4) |

High Eta OTL selection

high_eta(mgb_df, MCP, f, Training_all)

| Variables | Description |
| --- | --- |
| mgb_df | DataFrame/CSV containing detectable subsets for all OTLs |
| MCP | Results From MCP Results, used to determine the number of High Eta OTLs to test the same set of covered outages obtained from MCP |
| f | Output File: Used to Save Classification Results of High Eta test Created in main function |
| Training_all | Labeled Training Data, used in KNN classifier (component 4) |

Random OTL selection:

random_OTL(MCP, f, Training_all, LOIF)

| Variables | Description |
| --- | --- |
| MCP | Results From MCP Results, used to determine the number of Random OTLs to test the same set of covered outages obtained from MCP |
| f | Output File: Used to Save Classification Results of Random OTL test Created in main function |
| Training_all | Labeled Training Data, used in KNN classifier (component 4) |
| LOIF | Line Outage Impact Factors (Simply Used to Select a Random Sample From set of OTLs) |

Component 4: LOD classification experiment execution: Python function lod_exp_exec()

Runs KNN classification on a set of OTLs for a set of outages. The function is used in detectable subset generation in component 2 and OTL selection methods in component 3. In addition, we use MinMax Scalar to convert powerflow measurements to numbers between 0 and 1. Then we split Training Data to Create Testing Data, ensuring we get an equal number of samples for all scenarios being tested. We set up KNN classifier with k-neighbors, using uniform weights and auto method calculating distance using Minkowski formula. We then predict labels from testing data analyzing classification performance compared to true labels.

lod_exp_exec(Training_all, otl_col, outage_labels, k, output, f)

| Variables | Description |
| --- | --- |
| Training_all | Labeled Training Data, used in KNN classifier |
| otl_col | Column Labels of OTL(s) to filter Training Data |
| outage_labels | Row Labels of outages to filter Training Data |
| k | Number of k-neighbors used in KNN classifier Provided by user in main |
| output | If True, Print Classification Performance |
| f | Output File: Used to Save Classification Results Created in main function |